# TECHNICAL SPECIFICATION — OFFICE FIT-OUT

Project: Kingdom Tower Annex — Floor 12 Office Fit-Out

Location: King Fahd Road, Riyadh, KSA

Client: [Confidential]

Total Area: 650 m² (NLA) | Grade: Premium | Date: Q1 2025

## 1. SCOPE OF WORKS

This specification covers the complete interior fit-out of Floor 12 including all civil, architectural finishing, MEP, and FF&E works as described herein. Structural modifications are excluded unless explicitly noted.

### 1.1 Included Trades

Civil & Partition Works

Flooring & Floor Finishes

Ceiling Systems

Painting & Wall Finishing

Mechanical & Electrical (MEP)

Furniture, Fixtures & Equipment (FF&E)

### 1.2 Excluded Works

Structural modifications

Building facade works

External landscaping

IT infrastructure & cabling (by others)

Permits and authority approvals

## 2. MATERIALS SPECIFICATION

### 2.1 Flooring

| Area | Material | Size/Spec | Brand/Grade |
| --- | --- | --- | --- |
| Reception & Corridors | Porcelain Floor Tiles | 600x600mm, Rectified, Polished | RAK Ceramics / Porcelanosa |
| Open Plan Office | Raised Access Floor | 600x600, 150mm ht, 12kN/m² | Kingspan / Tate |
| Private Offices | Carpet Tiles | 500x500mm, 6.5mm pile, Loop | Interface / Shaw |
| Pantry & Support | LVT Vinyl Flooring | 4mm, 0.5mm wear layer | Karndean / Amtico |
| Board Rooms | Engineered Wood Flooring | 180x900mm, 14mm thick, Oak | Boen / Kahrs |

### 2.2 Partitions & Ceilings

Gypsum Board Partitions: 75mm metal stud, double skin 12.5mm moisture-resistant gypsum board, mineral wool infill for acoustic rating Rw 42dB minimum. Height: floor to structural slab (approx. 3.2m). All partitions to be fully coordinated with MEP services.

Glass Partitions: 12mm tempered clear glass, floor-to-ceiling, aluminium framing system (Schüco or equivalent). Manifestation to comply with SASO and building regulations.

Suspended Ceiling: Armstrong Ultima 600x600 mineral fibre tiles, exposed grid system, colour white. Grid to be fully coordinated with lighting, A/C, fire alarm, and sprinkler layout.

### 2.3 MEP Specifications

Air Conditioning: Daikin or Carrier split units, R-410A refrigerant. All units to be SASO-certified and comply with Saudi Energy Efficiency Program (SEEP) requirements. Minimum EER: 3.2.

Electrical: All works to comply with Saudi Arabian Standards (SASO) and IEC standards. Cables to be LSZH (Low Smoke Zero Halogen). Distribution boards to be fully labelled with load schedules provided.

Fire Alarm: Addressable system complying with NFPA 72 and Saudi Civil Defense requirements. System to be connected to building BMS. All devices to be listed/approved.

## 3. QUALITY STANDARDS

All materials and workmanship shall comply with the following standards:

SASO (Saudi Standards, Metrology and Quality Organization)

IEC / ISO standards where applicable

NFPA life safety codes

ASHRAE 90.1 for MEP energy efficiency

Manufacturer's written installation instructions

## 4. PROJECT CONSTRAINTS

| Constraint | Requirement |
| --- | --- |
| Access Hours | 7:00 AM – 5:00 PM, Saturday to Thursday |
| Noise Restrictions | No heavy demolition/drilling before 8:00 AM |
| Hoisting | Freight elevator available – max load 1,500 kg |
| Dust Control | Hoarding required on all occupied adjacent floors |
| Program | Target completion: 14 weeks from mobilization |